# Manufacturing SOP Checklist

# Scope & Purpose

To be used by the Supervisor to check off that all activities have been done by the Technician correctly and in the correct order.

# Entry to Makespace

- Outdoor clothing removed and stored in the ‘Dirty Goods In’ area, away from work tables

- Sign in

- “Dirty goods in” “Prep area” and “Production Area” are clearly designated (e.g. by tape) and understood by all personnel

TO OUTSIDE

CORRIDOR

KEY

Door

Work Table

Laser Cutter

Laser Cutter Computer

Other Equipment

Sink

FINISHED GOODS OUT

PREP AREA

CLEAN GOODS IN

CUTTING & ASSEMBLY

PACKAGING

CORRIDOR

DIRTY GOODS IN

LOGGING GOODS IN

STORAGE - LOGGED GOODS

UNPACKING/ CLEANING GOODS IN

STORAGE - NEW GOODS

Movement of personnel

# Preparation of work areas

- Prepare cleaning materials:

  - Regular water, detergent & cloths

  - EPA approved disinfectant solution & cloth

- Enter “Prep Area”:

  - Wash hands for 20s

  - Don surgical mask

  - Don gloves

- Clean “Production Area”:

  - Clean floor and work tables with regular cleaning materials

  - Disinfect work tables

  - Prep each tool/piece of equipment by wiping it with a cloth wetted with disinfectant , and placing it in the appropriate location for use

  - Check off each piece of equipment:

    - Laser Cutters wiped down - cover, and sides/edges that may be touched when cover is open

    - Laser Cutter bed -

    - Computer keyboard wiped down

    - Steel bars (weights) next to laser cutter (cleaned)

    - “Assembly Area” contains:

      - Cutting matt (cleaned)

      - Stanley knife (cleaned)

      - 1m metal ruler (cleaned)

      - Scissors (cleaned)

      - Stapler (cleaned)

      - Staples QUANTITY

      - Cardboard box labelled “Elastic Strips WIP” lined with bin liner. Bin liner labelled with date & initials of Technician in format “ MS-yy/mm/dd-’Initials’ “

      - Cardboard box labelled “Foam Strips WIP” lined with bin liner. Bin liner labelled with date & initials of Technician in format “ MS-yy/mm/dd-’Initials’ “

    - “Packaging Area”

      - Label maker (cleaned)

      - Individual product labels QUANTITY

      - Instructions For Use QUANTITY

      - Shipping labels QUANTITY

      - Plastic bags for packing QUANTITY

      - Stack of flat pack boxes QUANTITY

    - Bin for waste material (cleaned)

TO OUTSIDE

CORRIDOR

KEY

Door

Work Table

Laser Cutter

Laser Cutter Computer

Other Equipment

Sink

FINISHED GOODS OUT

PREP AREA

CLEAN GOODS IN

CUTTING & ASSEMBLY

PACKAGING

CORRIDOR

DIRTY GOODS IN

LOGGING GOODS IN

STORAGE - LOGGED GOODS

UNPACKING/ CLEANING GOODS IN

STORAGE - NEW GOODS

Flat-pack boxes

Plastic bags

Product Labels

Shipping Labels

Label Maker

WIP - Elastic

Cutting mat

Knife

Scissors

1m ruler

WIP - Foam

Visor Jig

Stapler

Staples

BIN

Weights

- Set up label maker to print labels with today’s date and the initials of the Technician and Supervisor

- Enter “Dirty Goods In”:

  - Clean floor and work tables with regular cleaning materials

  - Disinfect work tables

  - Doff gloves (bin)

  - Doff surgical mask (bin for washing)

# Logging incoming goods

*May be excluded if proceeding straight to step 4

- Unpackaged goods should be left in their boxes in the area marked “Storage - New Goods”

- Before handling goods:

  - Enter “Prep Area”

  - Wash hands for 20s

  - Don surgical mask

  - Don gloves

- Move goods to work table “Logging goods in”

  - Open box, but do not remove any plastic wrapping

  - Log contents in ‘Goods In/Out Log”

  - Repackage in box if necessary

- Place goods in “Storage - Logged Goods”

- *Doff gloves (bin)

- *Doff surgical mask (bin for washing)

# Preparation of incoming goods

*May be excluded if proceeding straight from step 3

- *Before handling goods:

  - *Enter “Prep Area”

  - *Wash hands for 20s

  - *Don surgical mask

  - *Don gloves

- Move goods from “Storage - Logged Goods” to work table “Unpacking/cleaning goods in”

  - Open box and remove plastic wrapping

  - Log contents in ‘Goods In/Out Log”

  - Transfer items to “Clean Goods In”

- Check raw materials

- Doff gloves (bin)

- Doff surgical mask (bin for washing)

# Production

- Enter “Prep Area”

  - Wash hands for 20s

  - Don surgical mask

  - Don gloves

# Preparation of Elastic & Foam

- Take elastic and foam raw material from “Clean Goods In” to “Cutting & Assembly”

- Cut elastic into 33 ± 1 cm strips and place in “Elastic Strips WIP”

- Cut foam into 23 ± 2 cm by 2.8 ± 0.5 cm strips and place in “Foam Strips WIP”. DO NOT remove adhesive material.

# Laser cutting visors

- Do not remove protective film

- Place material on laser cutter, weigh down the center of where each visor will be cut with steel blocks

- Laser Cutter settings: 20% power, 60mm/s for 60W laser

- Cut using the supplied .dxf file (visor PET nested.DXF)

- Place cut visors in “Cutting & Assembly”

# Assembly

- Peel back protective film on visor enough to attach foam and elastic (about 4cm from top of shield)

- Place in jig

- Remove adhesive from foam, use jig to stick foam centrally on top edge of shield

- Quality Control:

  - Reject foam pieces that are more than 2cm shorter than the foam positioning slot in the jig

  - Re-cut foam pieces that are too long or too wide

- Remove visor from jig

- Lay elastic parallel against top edge of visor

- Quality Control:

  - Reject elastic that is more than 1cm shorter than width of visor

- Add 2 staples approximately 0.5cm-1.5cm from side of visor to fasten the elastic to the plastic of the visor

- Place finished items in the “Packaging Area”

# Packaging

- Remove protective plastic from visor

- Quality Control:

  - Reject visors with scratches greater than 1cm in length in line of sight

  - Reject visors with defects on the outer edge that may scratch and damage skin

  - Reject visors with the foam/elastic attached to the bottom of the visor

  - Reject visors where the staples have closed improperly forming a hazard which could cause minor injury such as scratches to hand/face

  - Reject visors with less than 2 staples

- Place label on outside of visor, in upper right corner (so label is on the other side of the plastic to the foam, and does not impede vision)

- Place each visor in plastic bag

- Pack 24 bags into each box

- Label the box with shipping instructions

- Place box at edge of “Production Area” in “Goods Out Zone”

- Clear any debris/waste materials into the bin

- Leave “Production Area” through “Prep Area”

  - Doff gloves (bin)

  - Doff surgical mask (bin)

# Goods Out

- Without entering “Production Area”, take boxes from “Goods Out Zone”

- Ship

# Cleaning Up

- Without entering “Production Area”, remove and empty bin

- Leave bin outside “Production Area” to be cleaned and replaced at the start of the next day of production

# Non-conformances

- If there is any non-conformance to this SOP (for example, entering the “Production Area” during production without PPE) the following actions should be taken:

  - Log the non-conformance in “Non-Conformance Log”

  - Label affected boxes of finished product with “Non-Conformance Label”

- These actions should also be taken if it is not possible to follow this SOP due to external constraints