# General Device Description & Intended Purpose

https://www.ncbi.nlm.nih.gov/pmc/articles/PMC5015006/

“Face shields are personal protective equipment devices that are used by many workers (e.g., medical, dental, veterinary) for protection of the facial area and associated mucous membranes (eyes, nose, mouth) from splashes, sprays, and spatter of body fluids. Face shields are generally not used alone, but in conjunction with other protective equipment and are therefore classified as adjunctive personal protective equipment. “

# Specifications for Product

Product Requirements

- Face shield must have a single elasticated strap.

- Doffing can be accomplished with a single hand.

- Face shields must fit snugly to afford a good seal to the forehead area - forehead contact at least width of eyes. Contact surface soft foam or rubber, to reduce slippage during wearing.

- Visors must not significantly distort vision. Normal font size text should be readable whilst wearing them.

- Face shields should reach down to the chin and to the point of the ear,  and seal to the forehead.

- The face should be sufficiently rigid to not move around (creating noise or distortion) whilst walking.

- The edges and corners or both the headband and visor should not be sharp such that they could rip gloves or cause injury. The staples must not touch the skin.

- The visor should have sufficient clearance to the face for goggles and a face mask to be worn comfortably.

- Visors should be resistant to fogging, either through coatings or ventilation.

- The face shield is single use so does need to be sterilisable or resistant to cleaning products

For reference: https://docs.google.com/document/d/15k20PMe14_XQa-8kTA2G7nSIrF1bGSpc90_GM9H8yNI/edit